Venus dirigible project.

1 Introducing

1.1 Preface

1.2 Sphere of using

2 Outside of the dirigible

2.1 Analyzing the environment

2.2 Specialties based on the atmosphere

3 Construction

3.1 Main construction ideas: Prototypes, good and bad ideas

3.2 Choosing devices.

3.3 Description of the devices (power consumption, mass, volume)

3.3.1 Communication system devices

3.3.2 Power devices

3.3.3 Research devices

3.3.4 Life devices

3.4 Main concept of construction

3.4.1 Form of the dirigible

3.4.2 Air tank construction

3.4.3 Gondola construction

3.5 Material choosing

3.5.1 Air tank materials

3.5.2Framing materials

3.5.3 Gondola materials

3.6 Estimating approximate size

4 Finalizing the project

5 Counts

3.2.id1

1 Introducing

Before heading, I should say that it is not the whole project; this file just represents some of my thoughts and the vector of moving this project. Nowadays I am a bit further than it is written here, and I hope that if you are interested in it, I will continue to send my promotions of this project. Nowadays I’ve just made an aerodynamic tube for testing the air resistance coefficient of folded and unfolded version of dirigible for ending making program that represents coordinates speeds and the process of landing into the atmosphere in general. To add some more information, now I am in the process of making the 1/20 scale model, especially the unfolding system.

1.1 Preface

Before starting the project i was thinking of Mars rovers and other machines for planet exploration. And only half a year ago when I started to go deeper in the sphere of planet exploring, I understood that there is one planet that was a bit forgotten - Venus. So, I decided to make a planet exploring machine that would work at the Venus atmosphere longer than any other machines before. I understand that every planet requires its own way of exploring so based on the Venus conditions, I chose the dirigible.  In my mind, it should measure the temperature, the wind, and the chemical structure of the atmosphere. Also, I want to make it small for sending more than one machine at one time. If they are smaller and they can be sent in huge portions, it wouldn't cost too much money to fail the "landing" of one or two dirigibles.

1.2 Sphere of using

In general, I want to see this dirigible as a main research unit in the atmosphere. In opportunity we can send a lot of dirigibles, that could measure all the parameters that we need/ also cartography the surface of the planet with high frequency. Or even swing down some research modules that could pick up the ground probes of the planet.

Also, NASA announced DAVINCI mission and my machine can fly before the DAVINCI and help in landing calculation by the giving exact information from the landing scape, in the perspective it could cartography the landing place and look after the landing module from above. Also, it can help us understand Venus’s atmosphere better and measure the gradient of temperatures and other given parameters.

2 Outside of the dirigible

2.1 Analyzing the environment

As I said, I want to make a thing that’ll be useful for some rovers, for example, something that can measure the speed of wind, density of the atmosphere and pressure or cartography of the landscape of Venus with high frequency. It’ll not be difficult to make, because due to the Wikipedia and the book by V.I. Morozov “ACHIEVEMENTS OF PHYSICAL SCIENCES. THE ATMOSPHERE OF VENUS” there is an area with conditions close to Earths.

Between 55km and 45km pressure and temperature are quite normal and aren’t a big problem also in this area, density of the atmosphere measures between the Earths and 3 Earths, so we can make and test a model even without any additional installation or it would be not so difficult to make such installation. I’ll call that layer – “lifelayer”

So that makes me a chance to make a model of dirigible at earth to test its viability and opening system.

Also, the 45-55km height is good for carto graphing because it is 5 times closer to the surface of the planet than the lowest orbit of Magellan. That means that the frequency of cartographing will be much higher than Magellan’s. Even with much smaller volume and power consumption.

The big advantage of this height is that in approximation, the inside gas can be considered ideal. Because pressure is not high (less than 3atm) and the temperature is not high (less than 100C), the difference between our approximation and real gas will be less than 1%. That also really helps because I can use the Mendeleev—Clapeyron equation.

2.2 Specialties based on the atmosphere

The temperature between 304K and 382K means that by going deeper into the atmosphere we’ll need to cool down electronics. But due to the maximum temperature of electronics of 373K, we’ll need only 1 cooling circuit. Or just accumulate the cool and let the electronics work close to critical temperatures.

If we’re talking about accumulating cool and when the temperature rises to the maximum, just turn off electronics and start to raise the dirigible until normal temperatures.

For this way I need to have a layer with low thermal conductivity, that material should be really light too, and it should have resistance for acids, and it should have high melt or burn temperature (I think more than 500-600K).

3 Construction

3.1 Main construction ideas: Prototypes, good and bad ideas

I had a lot of ideas on how to make a dirigible. These are some of the most realistic:

1. The 1st idea is to make a dirigible that has no own gas, and flies on the atmosphere gas.

The idea was interesting because it doesn’t require any external gas flying from the Earth, also it is not scared of gas leakage. But it has some minuses. If we want to use the atmosphere as the filler of an air tank, we should make its density lower. To make it lower, we should make the quantity of substances lower or make both. But while lowering the quantity of substance, the pressure becomes lower. So, we need to raise it somehow. Pressure raises by decreasing the volume or by increasing the temperature, so we need to increase the temperature, because the raising force of Archimede depends on the volume, so it should be constant (we’re talking about flying in the certain layer). And if we count temperature of the gas, we’ll see that we’ll need temperature 1.4 times higher with outside pressure (if we count on the height of 44km and the useful weight of 1 kg from each m**3, the ideal variant), and if the pressure is, for example 80% of the outside pressure, the temperature will be 1.23 times higher than outsides (if we count on the height of 44km and the useful weight of 1 kg from each m**3, the ideal variant), so the temperature will be 190 celsium. But if we have inside pressure 80% of outside and the pressure of 0.65 outside, we’ll be able to fly at the height of 44km. But in this way the difference between outside and inside pressure will be around 0.9*10**5 Pa, which is a lot. So, on one hand we have a possibility not to bring a gas from earth, and on the second, we need to have lower pressure => we need a stable framing, and also, we need to heat up the gas that requires the huge amount of energy.

See the counts in the end 3.2.id1

2. The 2nd idea is to make a dirigible with one huge air tank filled with helium.

It is a good idea because helium has low destiny and we can fly in the life-layer without heating it or changing its pressure. But helium is not renewable so it is not so good to make because of that we can fly higher at first and after, during the leaking out of helium we’ll add atmosphere gas, that will require us to go deeper inside the atmosphere. But due to the Fick’s law equation and our approximation (Approximation consists in accepting the outside gas as ideal), there will be no leakage of helium if the pressure is the same as outsides (I really believe in that law but nevertheless I would analyze another variant for being sure that it will work). So that means that the minimal volume concentration of helium is 20% with the payload of 1kg, moreover the payload can be raised to 2kg per m**3 if the concentration is 60% and if we want the payload of 2.5kg, we only need the concentration of 80% and finally if concentration 100% we could bring with us 3 kg per m**3. So, we can just throw off some external furniture and go upper into the atmosphere. I understand that the lower concentration of helium slows it leakage out.

See the counts in the end 3.2.id1

3. The 3rd idea was to make a dirigible made of balloons filled with gas.

That was an interesting idea, because I was thinking of the dirigible made of separated parts that could fly independently and give more information than the obvious one. But it has a problem of low payload per volume because 2 balloons with the same volume as the one, would have more surface so they will weigh more, so their effectiveness is lower, but nevertheless idea of small, separated modules sounds good. Also, we'll need to have an antenna on each balloon part which will cost more and require a bigger volume of balloons.

4. The 4th idea was to make a dirigible with different length swinged research module.

This idea is interesting because it could measure parameters on different altitudes with only one dirigible, but it requires to have a long cable for swinged module and make a cooling system for electronics moreover long cable with weight on the end will make huge rotating moment. Long cable weighs a lot too and makes aerodynamics worse. But nevertheless, it is the only idea that can reach the surface of the planet, so maybe in the future I will think more about it.

5. The 5th idea was to make “mother” dirigible with her “sons”.

After some time, I was thinking of idea 3 and imagined how to use it. Separating modules is a good idea and I reworked it a bit. The new idea is interesting due to the fact that we will have only one main dirigible that could be able to fly for a really long period of time, while others could just fly around and if one of them fails, we could change them by losing not so much money. Also, that idea expands our research area at times and helps us with surface cartography. To say more, smaller dirigibles can even be without engines and just fly by the atmosphere winds. So, it allows us to have the maximum payload of volume from smaller dirigibles, with the possibility of expanding the research area and taking samples from Venus's surface and other lower layers of atmosphere.

I think that 5th idea is the best because it summarizes most of the advantages of previous ideas, it makes more opportunities of researching the planet, but it requires much more project work. And also, it will be a bit reworked during later project work.

3.2 Choosing devices.

Here I want to talk about the devices suitable for our main and additional dirigibles. For that, we need to recall the purpose of the project: firstly, it should deeply explore the atmosphere of Venus, get correct values of density, pressure, winds, and correctly analyze the chem structure of atmosphere. Because some days before I faced with the problem of an incorrect representation of the Venus cloud chem structure. In the 2025 Science Advances (https://www.science.org/doi/10.1126/sciadv.adg8826), it was said that our knowledge about Venus's atmosphere, especially clouds, was represented incorrectly. So, I want to understand is that really like that, because H2O including chemical compounds can lead us to the (close to) infinite oxygen by electrolysis (I’m not the fan of chemistry, but nevertheless I understand that we can have oxygen by electrolyzing the (H5O2)Fe(SO4)2·3H2O. With a huge amount of it we could even live flying in the Venus atmosphere. If it is like it is written, I want to use that gas for refilling the air tank of the dirigible. So, we need a thermometer, barometer, gas analyzer, density meter, and anemometer to research the parameters of the atmosphere. Also, we need devices for communication, navigation, and life-support devices such are power devices and main processors.

3.3 Description of the devices (power consumption, mass, volume)

Here I want to make some tips that could help me to make approximate mass of all the devices

3.3.1 Communication system devices

I want to have a long-distance radio transmitter, which could reach the signal for more than 1000km. I also don’t know circuit design and the theory of antennas on the necessary level, but I try to raise my knowledges, and I hope that it won’t be a huge problem to make such a transmitter because of absence of other signals in  Venus atmosphere and because of total clearness of atmosphere. (no wall or other barriers for signal) I found some information about Chinese radio transmitter which can send signal up to 100km with power consumption of 100W, so I would be rude and say that for 1000km we will have twice as big and 10 times powerful radio transmitter. According to that, the approximate size of it is 7kg 1kW and 140/83/172. Also, I have an idea of drop-down antenna for catching signals (see visualization in 3.4.4) which will have weight too, so I would take as an example my old village 4-meter length steel antenna which weights 20kg, I believe that antenna that would be bigger but made of composites will be at least the same weight.

So, the approximate mass of all the communicating system devices is 27 kg and the power consumption is 1000 W

3.3.2 Power devices

The only way for us to have energy is using solar panels, so we need to place them up on the top of air tank, the framing will hold these panels, also we need batteries. If we’re talking about solar panels, they can produce up to 400-500 W per 1 m**2 and with weight of approximately 10-15kg for that surface so we need to put at least 2 of them because we will have powerful radio transmitter to send signals to main dirigible. If we’re talking about battery, I don’t know how big it should be so I will put a 5kg external weight for battery.

So, the approximate mass of all the power devices is 25 kg and the power-giving is around 800-1000W

3.3.3 Research devices

As I said, we need a thermometer, barometer, gas analyzer, density meter and anemometer for researching what we need.

Thermometer. As an example, I brought the Palmer Model PST500 Solar Digital industrial thermometer, which has a weight of 1kg and has its own sun battery so I wouldn’t count the power consumption because the weight of solar battery is included in the weight of module.

Barometer. As an example, I brought one of Russian industrial digital barometers. It has an error rate of only 10Pa, which is low. And it has weight of only 3kg and power consumption of 15 W. Size: 200/210/80

Gas analyzer. Here I also brought one of Russian industrial digital Gas analyzers. It has weight of 8kg and power consumption of 60 W. Size: 400/300/150

Density meter. As an example, I brought LPGDi Digital lab Density meter, which has weight of 6.2kg and power consumption of 15 W. Size: 245/225/120

Anemometer. As an example, I brought one of Russian industrial digital Anemometers. It has a weight of only 2kg and power consumption of 20 W. Size: 300/150/150. Also, it must have a drive that would rotate it based on the wind.

So, the approximate mass of all the researching devices is 20.2kg and the power consumption is 110 W

Each of the dirigibles should have such a number of researching devices.

3.3.4 Life devices

Life devices are the main processor and pump, because without them we can’t navigate and pump outside/inside gas. Also, we need a small gas tank to make external pressure inside while unfolding. Usually pumps are not so big, from 3 to 5 kg and their power consumption is around 100W. For the main processor I would use the same as it was on ingenuity - just a phone processor, so its weight won’t be higher than 0.5kg and power consumption of 20W. And, finally, gas tank, its approximate weight is 15-20kg, for example I found one, which weighs 15 kg with volume of 10liters and max pressure of 150A(atmospheres). Of course, we need a smaller one so I would count it as 5 kg, because the main gas tank will fall onto the surface of the planet.

So, the approximate mass of all the life devices is 20 kg, and the power consumption is 120W

Summing up the results, I got the weight of all modules around 92kg. That is a lot, but I need to add more weight for the gondola itself and for wire and over things. So, I would count the approximate weight of the gondola and all other modules (except the air tank material) as 150kg.

3.4 Main concept of construction

Due to the fact that there will be 2 types of dirigibles, we’ll need 2 different constructions, but I understand that I won’t be able to make all on my own, so I will do only main thing for both dirigibles, but I’ll work over the smaller one, the “researching” dirigible. I want to make a prototype and test its viability on my own and model the landing of the dirigible.

3.4.1 Form of the dirigible

First, I want to talk about the form of smaller dirigibles. As I see, the smaller dirigibles don’t even need any engines and would just fly using wind, that means that we don’t need to have good aerodynamic shape for them, so, as we now, the sphere gives the most volume from its surface area, so it is ideal variant for dirigible because it gives us the least weight of shell. Of course, the sphere with a diameter less than 3m will have a “bigger” surface area (in m**2) than volume (in m**3), so it would be better if the sphere were bigger than 3m in the radius. To say more, if the density of air tank layer is more than 1 kg, the size of sphere can’t be less than 3 meters, with the payload of 1kg per 1 m**3 of gas.

Secondly, the main dirigible

3.4.2 Air tank construction

Air tank should be flexible to be folded in the spacecraft that’ll fly to Venus.

So, I had 4 ideas:

My 1st idea was to make an air tank that will open in the atmosphere and after that become firm. The only problem is finding material that will become firm after a long time, but I found such. It is a special glue that becomes firm after current passing through it.

| Advantages | Disadvantages |
| --- | --- |
| Can slow down herself during falling to the surface of the Venus | Hard to make air tank because of materials |
| Needs to be geometrically right formed to make current passing equally | Needs external batteries to make high current for making firm |
|  | Needs to be opened during falling into the atmosphere |
|  | Needs to deliver gas that will be inside air tank |
|  | Air tank is going to be heavy |

My 2nd idea was to open sphere on the orbit of the earth and fly with fully unfolded firm air tank to Venus. That idea has more possibilities because of its simplicity and lack of air resistance in space. Also I don’t need a special material for making gondola firm, because we can use even epoxy resin or some silicon sealants and glues. But the main probem of this idea, is landing because of high speeds and air resistance forse it will just burn in atmosphere, or we'll need a

| Advantages | Disadvantages |
| --- | --- |
| Easiness of construction, needs to be just soaked in sealants or glues | Needs time to be unfolded on the Earth’s orbit |
| Doesn’t need any external batteries | Harder to make it pass through the high layers of the atmosphere |
| Doesn’t need to be right-shaped | Air tank is going to be heavy |
| Less weight to deliver to Venus. |  |

The 3rd idea was to make a framed air tank with spring unfolding system

| Advantages | Disadvantages |
| --- | --- |
| Unfolds really fast. | Can easily jam |
| Weight less than previous variants | Hard to make and fold frame because of huge volume of unfolded gondola. |
| Doesn’t need any external modules |  |

And finally, 4th idea was to make an air tank fully soft and make higher pressure inside it to make it firm.

| Advantages | Disadvantages |
| --- | --- |
| Doesn’t need any component that will make it firm => less weight | Needs higher pressure inside than outside (for cancelling deformation by winds) => bigger gas leakage |
| Easy construction | Hasn’t any external layer (glue and sealants in other variants) to prevent gas leakage |
| Easy to fold and unfold |  |

Of course, there used to be some other ideas, but they were too crazy or had too many disadvantages, so I even didn’t mention them.

So, for transmitter I wanted to unite and bring only good sides from each of my ideas, but unluckily some of the disadvantages are still left. The middle concept is to make a soft air tank with epoxy resin or some other glue or sealant as drying framing. That would open in the Venus atmosphere

| Advantages | Disadvantages |
| --- | --- |
| Firm framing means that external pressure isn’t required inside gondola | Needs time to be unfolded |
| Not heavy because it has framing and the material of the air tank isn’t fully soaked in glue or sealant. | Hasn’t any external layer (glue and sealants in other variants) to prevent gas leakage P.S some pages later we’ll see that it’s not so bad |
| Doesn’t need to be right-shaped |  |
| Construction is not really hard |  |

And for researching dirigible I want to make a fully soft air tank.

3.4.3 Gondola construction

3.5 Material choosing

Also, we should think about the toxic atmosphere of Venus. That makes us choose the right materials just not for melting in the acid atmosphere. Especially carbon and sulfur acids. So, material of outside layer shouldn’t react with carbon and sulfur acids. This material should be good reflecting. 1st thing that comes into my brain is foil. It usually has a side layer of Al2O3, which has aluminum in the lowest oxidation degree and won’t react with acids in the atmosphere.

So, the main concept for the main dirigible is to make a gondola with special glue, resin or sealant framing that will unfold while flying to the Venus and become firm. Now I want to compare and choose materials for framing and for the main layer. And for the researching dirigibles I want to make

3.5.1 Air tank materials

Main layer material should be light and protect from acids as I said before. Also, it should be reflective. It can be multi-layered composite material that could stretch for folding it.

If we look at the construction of obvious dirigibles, we could see that they’re usually made of materials like cloth, nylon or polyester covered with polyvinyl chloride or polytetrafluoroethylene (Teflon) or foil.

1st is cloth.

| Advantages | Disadvantages |
| --- | --- |
| Light and soft. | less durable than nylon and polyester. |
|  | More expensive to produce and less accessible. |
|  | Good breathability. |
|  | Can be sensitive to moisture and dirt. |

2nd is nylon.

| Advantages | Disadvantages |
| --- | --- |
| High strength and tear resistance. | May be less UV resistant without treatment. |
| Lightweight and flexible. | Can absorb moisture, which can affect weight. |
| Good chemical resistance. |  |

3rd is polyester.

| Advantages | Disadvantages |
| --- | --- |
| High strength and wear resistance. | May be less flexible compared to nylon. |
| Resistant to moisture and chemicals. |  |
| May be less breathable tha nylon |  |
| Good UV resistance. |  |

I think that polyester looks like a good variant, but due to its less flexibility, I will also look at nylons. But as i know, nowadays nylon is used for covering dirigibles, so  i will choose it, but i will also try other materials while making prototypes.

Now let’s talk about cover materials; I thought about only 3 of them.

1st is PVC (polyvinyl chloride) or other forms with different chem addings.

| Advantages | Disadvantages |
| --- | --- |
| High strength and tear resistance | May lose flexibility at low temperatures |
| May be less UV resistant without additives |  |
| Resistant to moisture and chemicals |  |
| Good tightness |  |

2nd is polytetrafluoroethylene (Teflon).

| Advantages | Disadvantages |
| --- | --- |
| Excellent chemical resistance | Higher cost compared to PVC |
| High temperature resistance | Less durable and flexible than PVC |
| Low friction and good self-cleaning ability | Difficult to process and combine with other materials |

3rd is foil.

| Advantages | Disadvantages |
| --- | --- |
| Lightness and high reflectivity. | Low strength and resistance to mechanical damage. |
| Good tightness. | May be susceptible to corrosion and moisture damage. |
| Low cost. | Limited durability. |

The best option is, of course, PVC, but if we cover foil with another transparent layer, disadvantages disappear. So, we can use more than one cover layer and make 3 or more layers composite.

For example, polyester-foil-PVC or nylon-foil-PVC. These composites are quite good, they reflect sunlight perfectly, also they have resistance to acids and chemicals, flexible, and look like a brilliant variant.

3.5.2 Framing materials

I have 3 variants: glues, sealants and resins.

The 1st variant are glues.

| Advantages | Disadvantages |
| --- | --- |
| Doesn’t heat during congealing | Congeals after contacting with air |
|  | automatically congeals during opening |
|  | Usually is not hard |
|  | Congeals fast |

2nd variant are sealants.

| Advantages | Disadvantages |
| --- | --- |
| Congeals slower than glue | Congeals after contacting with air |
| Doesn’t heat during congealing | Usually is not really hard |
|  | Will automatically congeal during opening |

3rd variant are epoxy resins.

| Advantages | Disadvantages |
| --- | --- |
| The strongest material | Needs to be mixed before using |
| We have ideal conditions for congealing epoxy resins | Heats during congealing |
| Won’t automatically congeal during opening | Needs to have temperature of 50-60c |

There are a lot of types of epoxy resins. And I made table of them:

| Type | Melting temperature | Compressive strength (MPa) | Fracture stiffness (MPa) | Density (g/cm3) | Solidification temperature (°C) |
| --- | --- | --- | --- | --- | --- |
| Bisphenol A-based epoxy resin | 150-200 | 70-100 | 80-120 | 1.15-1.20 | 50-80 |
| Bisphenol F-based epoxy resin | 160-220 | 220 80 | 90-130 | 1.20-1.25 | 60-90 |
| Epoxy resin based on aliphatic compounds | 50-100 | 30-60 | 60 40 | 1.05-1.15 | 40-70 |
| Epoxy resin based on novolac resins | 120-180 | 60-90 | 70-110 | 1.15-1.20 | 50-80 |
| Low viscosity Epoxy resin | 60-120 | 40-70 | 50-90 | 1.05-1.15 | 30-60 |

According to the table we can analyze which variants are the most perspective according to their melting temperature and other parameters.

Melting temperature should be higher than 100°C, the maximum temperature where we would fly. The second parameter is density; it also should be as low as possible. And the 3rd is strength, but now, unluckily, we can’t choose the material according to this parameter because I haven’t counted loads on the frame.

But according to previous 2 parameters, the 1st place is Bisphenol A-based epoxy resin the 2nd is Bisphenol F-based epoxy resin and 3rd is Epoxy resin based on novolac resins

3.5.3 Gondola materials

Gondola should be light strong and gas-tight, I see it as a donut-shaped casing with all the equipment inside. Also, material should have good chem-resistance. According to that parameters I would mention some composites, for example carbon fiber, which is light, strong and with a small layer of foil becomes chem-resisted.

3.6 Estimating approximate size

Here I want to count the approximate size of the research dirigible and how many of them can be sent to the Venus at one time. Also here is finally would be the whole model of the dirigible. For that I made a program using Python, that gives us the radius of a dirigible air tank. This program just solves the cube equation of floating in the atmosphere by the Cardano formula.

Where layer density is the weight of 1m ** 2 of main layer of air tank, payload is the mass of gondola and tools and payload of 1m **3 is the mass that can be lifted by 1 m ** 3 of helium (it can be count/chosen by interactive visualization graph 3.2.id1).

Also, I’m making a program that will count the speeds temperatures and other parameters of dirigible during landing, considering the force of air resistance, force of Archimede and other parameters such are existance of parachute system and opening of the dirigible in lower layers basing on the 1st space speed of Venus, angle of entry into the atmosphere and the variability of the dependence of air resistance on speed.

4 Finalizing the project

To end this part of the project, I want to describe the main construction that I invented and started to make it real. The final construction that I invented consists of the main middle pipe framing with hidden antenna inside.

antenna inside and folded solar panels. Both panels and antenna in my mind will open by the force of parachute, after disconnecting it will fly straight up, and it will pull out the “furniture”.

Here you can see a prototype that can be unfolded by straps of parachute (Pic.2). That system has some advantages such are absence of any motor or everything that will open this system, also there are

Pic.2

no external moving parts that can break down jam if they are oiled, also the system becomes firm after opening. Also, the main plus is having a huge antenna after opening.  I think that I should mention why I used pipe framing. Pipes are badly bendable, also, they hold tension and compression loads well, so for my framing where the only 2 forces that are acting on the frame are the force of weight (mg) and the force of air resistance of parachute (and force of air resistance of  the apparat which is much lower than the force of parachute), so the only alleged loads are tension and compression loads because both of forces are acting along one line. To say more, inside the pipe I wanted to hide an antenna for easier transporting, because with an outside antenna, transporting becomes harder due to the raising of the volume parameters. So, in my opinion, this idea looks good and even better that I can model this system and print it for testing to show the idea better, I made a picture of how it will look like. (Pic.3).

Pic.3

Using the program, I count the maximal value of acceleration during falling it will be at the moment of opening parachute. By the 2nd law of Newton there will be 2 forces that act along the line of framing pipe so, the force of compression will be the mass multiplied with the sum of maximal acceleration and g. Using this we can count the approximate strength limit and the approximate thickness and mass of framing.

In my situation it turned out that I need a carbon pipe framing with thickness of walls around 1.6-2.0mm and with approximate size of air tank, I got a mass of 15-19kg (diameter of the pipe is 0.3m). Quite good, at first, I thought that it would be much thicker. Add some more mass for mounting points of solar batteries and rails, and the approximate mass is going to be 15-25kg.

If we’re talking about approximate mass, I should say that my approximate counts show that final mass with heat resistant nose and parachutes will be less or around 500kg for 1 dirigible. That means that at one time we can send at least 3 of such machines.

All the counts are shown in program that I coded, to start it read the DocProg file.

5 Counts

Nowadays I’m making file with all formulas and counts using tech. I’ll send it after ending it with all formulas. So, see the counts in file “ProjectFormulas”.

3.2.id1

Of course, this graph has a lot of problems connected with errors because of inaccuracy of regression and because we use the equations of ideal gas for non-ideal, especially on low heights, so this graph was just made for visualization and represents approximate values, that can be close to truth.

https://www.desmos.com/calculator/8rvn2elodm

Ending.

To end up, I think that I did a small amount of work that is a first step for making this model real. The next step is to make an experiment on my self-made air turbine and end program using the values from experiments. Parallelly I’m testing covering materials in my school chem laboratory (trying to simulate the Venus conditions from life layer).

So, I want to represent first mod and prototype of pipe framing up to the end of February and finally end the 1/20 scale model.